# Temperature Data Entry and QA/QC

# Standard Operating Procedures

- Logger Deployment:

HOBOware temperature loggers are deployed on every CREMP monitoring site to record temperature over time. The devices are manually deployed and collected each year by a dive team and are immediately replaced with a new logger to prevent gaps in data coverage. While there are many factors out of our control that can introduce error or create gaps in our survey (loggers lost, stolen, battery dies, etc.), the procedure below has been designed to minimize loss and increase the consistency of our results.

  - Open HOBOware:

  - Insert U22 HOBO temperature logger into the Shuttle, aligning the nub on the logger with the notch in the shuttle (there is also an arrow on the logger pointing at the nub).

  - Squeeze lever towards shuttle; if successful green “OKAY” light will turn on and HOBOware will automatically recognize it

  - Click the logo for “Launch Device”

  - Select the device (the only option should be for the logger you plugged in)

  - Fill out the “Launch Logger” page that pops up:

- Rename logger to the station and year it will be deployed at, i.e. Wonderland_1819

- Check that the battery state is good

- Configure sensors to log temperature

- Set logging interval to 1 hour

- Start logging at the date and time you will need to use the logger. Loggers can be launched weeks in advance; just make sure you err on the side of launching the logger too early and not too late

  - Record the serial number, site, and launch date in your temperature log and the TempLoggerLog.xls file

- Prepare logger for the field:

  - Place the logger inside the white boots and put the white cap on

  - Thread two zip ties through the hole on the end and tape them flat against the logger using duct tape

  - Label the tape with the Site and Station using a Sharpie. Stations are listed on the Temperature Data at a Glance file

  - Alternatively, if no boot is available, put a small black cap on the logger and wrap the loggers in electrical tape, or saran wrap and then duct tape, then duct tape the zip ties and label

- Deployment:

  - Loggers float – zip them up in your BC pocket or secure them inside a mesh bag

  - Remove the old logger from the stake (I prefer the twist and pull over a knife), put it in your pocket (because they float!) and secure the new logger to the stake with the two zip ties

  - Take a picture of the logger immediately after deployment for a time stamp for the end time of the old logger and the start time of the new.

- Temperature Database Management:

  - After recovery from the field, download temperature data using HOBOware

  - Insert U22 HOBO temperature logger into the Shuttle aligning the nub on the logger with the notch in the shuttle (there is also an arrow on the logger pointing at the nub)

  - Squeeze lever towards shuttle, if successful green “OKAY” light will turn on

  - Click the logo for “Readout Device”

  - Stop logging

  - Save the hobo File using “SiteNameLast2DigitsOfStartYear_Last2DigitsOfEndYear”, for example “Thor_1819.hobo”

  - Under Plot Setup, make sure Temp Series is selected and click “Plot”. You can now see the temperature data portrayed as a graph. To

  - Using the trimming tool and the deployment/retrieval logs, enter in the desired date range for the data, and identify any anomalies that may need to be trimmed off at the beginning during the acclimation period.

  - To save it into a csv, select “File” then “Export Table Data”, then “Export”.  Export the file with csv as the chosen file type.

- Temperature Database Management:

As of 2026, the temperature data is read in, manipulated, and cleaned in R (Version: 4.5.2 ([Not] Part in a Rumble) 2025-10-31), and edited and accessed through RStudio (Version: 2026.01.0+392).

For our csv files exported from HOBOware, some of the columns need to be separated out into our desired format (like date and time), while others simply need to be added (conversion from °C to °F).

The scripts used to clean all temperature data have been automated and can be re-run (with updated year’s data added to the correct folder). These scripts can be found on FWRI CRP's Github Organization Temperature Repository.

Between the Florida Keys National Marine Sanctuary (FKNMS) and Dry Tortugas National Park (DTNP), there are a total of 15 Quarto documents that have been used to clean data in the past. They will not be summarized here as that would greatly increase the length and complexity of this SOP. Rather, a single cleaning script for both the FKNMS and DTNP can be found in the “Temperature_Data_SOP” folder, that also contains a copy of this document and one cleaning script for the Southeastern Florida (SECREMP) temperature data. These are the files that should be used to clean the CSVs from HOBOWare.

Any questions regarding the structure or function of the cleaning code can be directed to the current database manager, Maddy Enda (email: Madison.Enda@MyFWC.com).

  - Metadata: Variable Definitions

This table shows the column names of the CSV files, their desired data types, and a brief description.

| Variable | Data Type / Unit | Description |
| --- | --- | --- |
| observation_id | Integer | Unique identifier for each individual observation that is recorded |
| date | Date (yyyy-mm-dd) | The date the measurement was recorded on site |
| time | HMS (standard military time /00:00:00) | The time of day that the measurement was recorded on site |
| site_id | Integer | A unique identifier assigned to each of the survey sites. The ID is serial through the Florida Key sites, Southeast Florida sites, and Dry Tortugas National Park. |
| site_name | Character | The name of the location where the surveys occur and where sensors are located (survey sites) |
| temp_C | Numeric (degrees Celsius/°C) | The temperature recorded by the device |
| temp_F | Numeric (degrees Fahrenheit/°F) | Converted from the recorded temperature in Celsius |

- Broken or Unreadable Logger? All is not lost!

  - If you recover a logger from the field that is either completely dead or has low battery and cannot read out the data, send the logger to Onset Computer Corp for data recovery (data back service)

  - Contact Onset using their live chat on www.onsetcomp.com or email Paula Murphy at Paula_Murphy@onsetcomp.com

  - Onset will walk you through the service, but you will send the broken logger to them, and they will email you the raw HOBOware data file and scrap the logger

- Mail to Onset Computer Corp: 470 MacArthur Blvd, Bourne, MA 02532

- Cost for data back service is $29.00

  - New loggers can be purchased at a discounted rate of $95.00 per logger